# Optical flow and keypoint tracking

- Given two subsequent frames of a video, the optical flow field indicates the apparent motion of each pixel

- If we have more than two frames, we can track features from one frame to the next by following the optical flow

# Motion is a powerful perceptual cue

- Sometimes, it is the only cue

- Even “impoverished” motion data can evoke a strong percept

# Uses of motion in computer vision

- 3D shape reconstruction

- Object segmentation

- Learning and tracking of dynamic models

- Event and activity recognition

# Preview: Structure from motion

- Given a set of corresponding points in two or more images, compute the camera parameters and the 3D point coordinates

# Keypoint tracking

# Motion field

- It is the projection of the 3D scene motion into the image

# Optical flow

- It is the apparent motion of brightness patterns in the image

- Ideally, optical flow would be the same as the motion field

- Have to be careful: apparent motion can be caused by lighting changes without any actual motion

  - Think of a uniform rotating sphere under fixed lighting vs a stationary sphere under moving illumination

# Estimating optical flow

- Given two subsequent frames, estimate the apparent motion field u(x,y) and v(x,y) between them

- Key assumptions

  - Brightness constancy: projection of the same point looks the same in every frame

  - Small motion: points do not move very far

  - Spatial coherence: points move like their neighbors

# The brightness constancy constraint

- Brightness constancy equation:

[Ix, Iy are the brightness gradients in their corresponding directions]

- Linearizing the right side using Taylor expansion:

Derivation with respect time…

- How many equations do you have per pixel?

  - One equation, two unknowns

- What does this constraint mean?

  - The dot product is the value of the projection of the shortest vector to the longest vector

- The component of the flow perpendicular o the gradient (i.e., parallel to the edge) is unknown

# The aperture problem

# The barber pole illusion

# Solving the aperture problem

- How to get more equations for a pixel?

- Spatial coherence constraint: pretend the pixel’s neighbor have the same (u,v)

  - E.g., if we use a 5x5 window, that gives us 25 equations per pixel

- Least squares problem

- When is it solvable?

  - What I the window contains just a single straight edge?

  - When you have at least two linearly independent equations

# Conditions for solvability

- Bad case: single straight edge

- Good case

# Lucas-Kanade flow

Linear least squares problem

Solution given by (ATA)b = ATb

The summations are over all pixels

- Recall the Harris corner detector: M = ATA is second moment matrix

- We can figure out whether the system is solvable by looking at the eigenvalues of the second moment matrix

  - The eigenvectors and eigenvalues of M relate to edge direction and magnitude

  - The eigenvector associated with the larger eigenvalue points in the direction of fastest intensity change, and the other eigenvector is orthogonal to it

# Recall: second moment matrix

# Uniform region

- Gradients have small magnitude

- Small lambda1 and lambda2

- System is ill-conditioned

# Edge

- Gradients have one dominant direction

- Large lambda1, small lambda2

- System is ill-conditioned

# High-texture or corner region

- Gradients have different directions, large magnitudes

- Large lambda1, large lambda2

- System is well-conditioned

# Example of optical flow estimation

From the previous “sequence of images”

# Multi-resolution estimation

# Coarse-to-fine optical flow estimation

# Iterative Refinement

- Iterative Lukas-Kanade algorithm

  - Estimate displacement at each pixel by solving Lucas-Kanade equations

- Warp I(t) towards I(t+1) using the estimated flow field

  - Basically, just interpolation

- Repeat until convergence

# Optical flow results

# Errors in Lucas-Kanade

- The motion is large (larger than a pixel)

  - Coarse-to-fine estimation

  - Iterative refinement

  - Exhaustive neighborhood search (feature matching)

- A point does not move like its neighbors

  - Motion segmentation

- Brightness constancy does not hold

# Large displacement optical flow

Start with something similar to Lucas-Kanade

- Gradient constancy

- Energy minimization with smoothing term

- Region matching

- Keypoint matching (long-range)

# Feature tracking

- If we have more than two images, we can track a feature from one frame to the next by following the optical flow

- Challenges

  - Finding good features to track

  - Adding and deleting tracks

# Shi-Tomasi feature tracker

- Find good features using eigenvalues of second-moment matrix

  - Key idea: good features to track are the ones whose motion can be estimated reliably

- From frame to frame, track with Lucas-Kanade

  - This amounts to assuming a translation model for frame-to-frame feature environment

- Check consistency of tracks by affine registration to the first observed instance of the feature

  - Affine model is more accurate for larger displacements

  - Comparing to the first frame helps to minimize drift

# Tracking example

# Summary of KLT tracking

- Find a good point to track (Harris corner)

- Use intensity second moment matrix and difference across frames to find displacement

- Iterate and use coarse-to-fine search to deal with larger movements

- When creating long tracks, check appearance of registered patch against appearance of initial patch to find points that have drifted

# Implementation issues

- Windows size

  - Small window more sensitive to noise and may miss larger motions (without pyramid)

  - Large window more likely to cross an occlusion boundary (and it’s slower)

  - 15x15 to 31x31 seems typical

- Weighting the windows

  - Common to apply weights so that center matters more (e.g., with Gaussian)

# Why not just do local template matching?

- Slow (need to check more locations)

- Does not give subpixel alignment (or becomes much slower)

  - Even pixel alignment may not be good enough to prevent drift

- May be useful as a step-in tracking if there are large movements

# Summary

- Major contributions from Lucas, Kanade, Shi, Tomasi

  - Tracking feature points

  - Optical flow

- Key ideas

  - By assuming brightness constancy, truncated Taylor expansion leads to simple and fast patch matching across frames

  - Coarse-to-fine registration